# Preface

Sutter Instrument Company has created this LabView program as an object-oriented interface to control the MP-285 micromanipulator via a host computer.  Use this software as a template for creating your own programs.  This software is intended for users who are already familiar with using the front panel of the MP-285 to configure setups and programs for using the robotics in experimentation.

The LabView MP-285 Controller Package                                 Version 1.00

This software is designed for configuring and executing micromanipulator movements and controlling various functions of Sutter Instrument Company’s MP-285.

You should have LabView 5.0 or better running on your computer and you should have some experience with the Labview programming environment.

The User Interface

Be sure that the port number is set correctly so that your computer is communicating with your MP-285.

Choose the velocity, in micrometers per second, at which you want your manipulator to move.

There are three control boxes, each corresponding to one of the axes of the MP-285.  The user enters the desired position in micrometers that he/she wants the manipulator to move to, and then runs the VI.